# HW#4 Tianpu Zhao

(1)

The Verilog files are attached in the canvas submission. Here is a copy-paste version. The .sdc file is not included in the submission (submission type only allows .v)

The module file: Q1_ALU.v

//============================================================

// 8-bit ALU (unsigned inputs), F[2:0] selects operation

// F=000: Q=a+b              (Cout = carry-out)

// F=001: Q=a-b              (Cout = borrow   ; 1 if a<b, else 0)

// F=010: Q=a OR  b

// F=011: Q=a AND b

// F=100: Q=a XOR b

// F=101: Q=NOT a

// F=110: Q=arithmetic left  shift a by 1  (<<1; LSB=0)

// F=111: Q=arithmetic right shift a by 1  (>>1; MSB holds sign bit a[7])

//============================================================

`timescale 1ns/1ps

module alu8 (

input  wire [7:0] a,

input  wire [7:0] b,

input  wire [2:0] F,

output reg  [7:0] Q,

output reg        Cout // carry-out for add, borrow for sub

);

// Internal 9-bit adders for proper carry/borrow handling

wire [8:0] add9 = {1'b0, a} + {1'b0, b};

wire [8:0] sub9 = {1'b0, a} + {1'b0, ~b} + 9'b000000001;

always @* begin

Q    = 8'h00;

Cout = 1'b0;

case (F)

3'b000: begin // a + b

Q    = add9[7:0];

Cout = add9[8]; // true carry-out

end

3'b001: begin // a - b

Q    = sub9[7:0];

Cout = ~sub9[8]; // borrow = NOT(carry-out of a + ~b + 1)

end

3'b010: begin // a OR b

Q    = a | b;

Cout = 1'b0;

end

3'b011: begin // a AND b

Q    = a & b;

Cout = 1'b0;

end

3'b100: begin // a XOR b

Q    = a ^ b;

Cout = 1'b0;

end

3'b101: begin // NOT a

Q    = ~a;

Cout = 1'b0;

end

3'b110: begin // arithmetic left shift by 1 (same as logical for 2's comp)

Q    = {a[6:0], 1'b0};

Cout = 1'b0;

end

3'b111: begin // arithmetic right shift by 1 (preserve sign bit)

Q    = {a[7], a[7:1]};

Cout = 1'b0;

end

default: begin

Q    = 8'h00;

Cout = 1'b0;

end

endcase

end

endmodule

The testbench: Q1_tb_ALU.v

`timescale 1ns/1ps

module tb_alu8;

reg  [7:0] a, b;

reg  [2:0] F;

wire [7:0] Q;

wire       Cout;

alu8 dut(.a(a), .b(b), .F(F), .Q(Q), .Cout(Cout));

// Reference model for checks

function [8:0] add9(input [7:0] x, input [7:0] y);

add9 = {1'b0,x} + {1'b0,y};

endfunction

function [8:0] sub9(input [7:0] x, input [7:0] y);

sub9 = {1'b0,x} + {1'b0,~y} + 9'd1; // x - y

endfunction

task check(input [7:0] ta, input [7:0] tb, input [2:0] tF);

reg [7:0] expected_Q;

reg       expected_C;

reg [8:0] tmp;

begin

a = ta; b = tb; F = tF;

#1;

case (tF)

3'b000: begin

tmp = add9(ta,tb);

expected_Q = tmp[7:0];

expected_C = tmp[8];

end

3'b001: begin

tmp = sub9(ta,tb);

expected_Q = tmp[7:0];

expected_C = ~tmp[8];

end

3'b010: begin

expected_Q = ta | tb;

expected_C = 1'b0;

end

3'b011: begin

expected_Q = ta & tb;

expected_C = 1'b0;

end

3'b100: begin

expected_Q = ta ^ tb;

expected_C = 1'b0;

end

3'b101: begin

expected_Q = ~ta;

expected_C = 1'b0;

end

3'b110: begin

expected_Q = {ta[6:0],1'b0};

expected_C = 1'b0;

end

3'b111: begin

expected_Q = {ta[7], ta[7:1]};

expected_C = 1'b0;

end

endcase

if (Q!==expected_Q || Cout!==expected_C) begin

$display("FAIL  F=%03b  a=%0d (%08b)  b=%0d (%08b)  -> Q=%0d (%08b) Cout=%0b  expected_Q=%0d (%08b) expected_C=%0b",

tF, ta, ta, tb, tb, Q, Q, Cout, expected_Q, expected_Q, expected_C);

end else begin

$display("PASS  F=%03b  a=%0d (%08b)  b=%0d (%08b)  -> Q=%0d (%08b)  Cout=%0b",

tF, ta, ta, tb, tb, Q, Q, Cout);

end

end

endtask

initial begin

$display("---- 8-bit ALU Test ----");

// ADD tests (including 250+5 & 250+6)

check(8'd12,  8'd34, 3'b000);

check(8'd250, 8'd5,  3'b000); // expect Cout=0

check(8'd250, 8'd6,  3'b000); // expect Cout=1

// SUB tests (7-3 -> borrow 0; 3-7 -> borrow 1)

check(8'd7,   8'd3,  3'b001);

check(8'd3,   8'd7,  3'b001);

// Logic ops

check(8'b00101110,  8'b00011010, 3'b010); // OR

check(8'b00101110,  8'b00011010, 3'b011); // AND

check(8'b00101110,  8'b00011010, 3'b100); // XOR

check(8'b00101110,  8'b00011010, 3'b101); // NOT

// Shifts (show arithmetic right keeps MSB)

check(8'b10110100,  8'b01010101, 3'b110); // <<1

check(8'b10110100,  8'b01010101, 3'b111); // >>>1 (MSB stays 1)

$display("All tests done.");

#5 $finish;

end

endmodule

The sdc file (one-liner)

set_max_delay 1.0 -from [all_inputs] -to [all_outputs]

(2)

Waveforms:

Included are examples of F = 000 (summation):

Case 1: a = 12, b = 34, Cout = 0, Q = 46

Case 2: a = 250, b = 5, Cout = 0, Q = 255

Case 3 (in the middle): a = 250, b = 6, Cout = 1, Q = 0

F = 001 (difference):

Left: a = 7, b = 3, Cout = 0, Q = 4

Right: a = 3, b = 7, Cout = 1, Q = 252

Left: F = 010 (a or b)

a=00101110, b=00011010

Q=0011110

Right: F = 011 (a and b)

Same set of a, b

Q = 00001010

Left: F=110 (arithmetic left shift)

a=10110100 -> Q=01101000

Right: F=111 (arithmetic right shift)

a=10110100 -> Q=11011010

Left: F=100 a xor b

a=00101110, b=00011010 -> Q=00110100

Right: F=101 not a

a=00101110 -> 11010001

(3)

Total area = 396.673, cell count 160

Slack = 556ps

The area report area.rpt

============================================================

Generated by:           Genus(TM) Synthesis Solution 18.14-s037_1

Generated on:           Oct 30 2025  02:31:05 pm

Module:                 alu8

Operating conditions:   typical

Interconnect mode:      global

Area mode:              physical library

============================================================

Instance Module  Cell Count  Cell Area  Net Area   Total Area

--------------------------------------------------------------

alu8                    160    161.728   234.945      396.673

Timing report timing.rpt

============================================================

Generated by:           Genus(TM) Synthesis Solution 18.14-s037_1

Generated on:           Oct 30 2025  02:30:39 pm

Module:                 alu8

Operating conditions:   typical

Interconnect mode:      global

Area mode:              physical library

============================================================

Path 1: MET (556 ps) Path Delay Check

Startpoint: (F) b[1]

Endpoint: (F) Q[7]

Capture    Launch

Path Delay:+    1000         -

Drv Adjust:+       0         0

Arrival:=    1000

Required Time:=    1000

Data Path:-     444

Slack:=     556

Exceptions/Constraints:

max_delay             1000            constraints.sdc_line_1

#----------------------------------------------------------------------------------------

# Timing Point   Flags    Arc   Edge   Cell     Fanout Load Trans Delay Arrival Instance

#                                                      (fF)  (ps)  (ps)   (ps)  Location

#----------------------------------------------------------------------------------------

b[1]           -       -      F     (arrival)      4  6.4     0     0       0    (-,-)

g5672__3772/ZN -       A2->ZN R     NOR2_X1        2  3.1    22    32      32    (-,-)

g5633__1840/ZN -       A1->ZN R     OR2_X1         1  1.2     6    26      57    (-,-)

g5614__2391/ZN -       A1->ZN R     AND2_X1        3  5.8    17    40      98    (-,-)

g5599__1857/ZN -       B1->ZN F     OAI21_X1       3  4.5    16    23     120    (-,-)

g5589__5266/ZN -       B1->ZN R     AOI21_X1       3  5.7    40    49     169    (-,-)

g5586/ZN       -       A->ZN  F     INV_X1         2  2.8    12    13     183    (-,-)

g5581__9682/ZN -       B1->ZN R     AOI21_X1       1  2.0    23    28     211    (-,-)

g5578/ZN       -       A->ZN  F     INV_X1         1  1.7     7     9     220    (-,-)

g5569__7344/ZN -       B1->ZN R     AOI21_X1       2  3.8    31    36     257    (-,-)

g5568/ZN       -       A->ZN  F     INV_X1         2  2.8    10    13     269    (-,-)

g5560__5953/ZN -       B1->ZN R     AOI21_X1       2  3.8    31    37     306    (-,-)

g5557/ZN       -       A->ZN  F     INV_X1         2  2.9    10    13     319    (-,-)

g5556__1309/ZN -       A1->ZN F     AND3_X1        1  1.8     7    31     350    (-,-)

g5551__1474/ZN -       B->ZN  R     AOI211_X1      1  2.0    39    65     415    (-,-)

g5540__8780/ZN -       B1->ZN F     OAI221_X1      1  0.3    16    29     444    (-,-)

Q[7]           <<<     -      F     (port)         -    -     -     0     444    (-,-)

#----------------------------------------------------------------------------------------

(4)

Gate-level netlist file – see Q1_ALU_syn.v submitted

The waveform (all the results are shown in one figure)